47T Sprocket

This year’s drivetrain goal was to optimize the system through weight optimization. Our previous 47T sprocket worked however its bulky steel construction made the part very heavy. Additionally the small radius at which the bolts where placed led to a higher stress on the bolts explained by the equation F = M/r, where a smaller radius means a higher shear force. Logically, the weight of the Sprocket-Hub assembly can be optimized through increasing the amount of aluminum and decreasing the amount of steel given that 7075 T6 aluminum’s density is 2.81 g/cm³ compared to that of 4340 steel at 7.85 g/cm³. The first step toward optimization was figuring out how far out I could place my bores, let’s call this the bore diameter. I used four bores simply because last year’s assembly used four so four bolts is my baseline. To figure out how far out I needed to place the bolts, I needed to make sure that the sprocket hub would clear the chain plates of the engaged roller chain. Using the ISO 606, 2015 Standard I created a 47 Tooth sprocket blank. From here I sketched out the rollers and the expected height of the chain plate, and added ~4 mm of additional clearance. Knowing the radius that had to be cleared I now had the maximum diameter possible for my sprocket hub Dmax. My bore radius was determined by having a 15mm gap between the outer edge of the maximum diameter of the sprocket hub and the bore radius; this bore radius ended up being 70 mm or 140 mm in diameter.

Hub Topology Optimization

To reduce weight it is necessary to optimize the topology of the sprocket hub. To optimize topology it is necessary to figure out the boundary conditions and loading conditions.

How was this done?

We know that the motor generate a maximum torque of 107N*m. We have the 10T sprocket attached to the motor has a pitch circle diameter of 41mm. We know F=M/r so 2000 * 207  / 41 equals the chain pull force. The chain also experiences centrifugal forces which adds to the chain tension this is given by F = mass per unit length of chain * velocity of small sprocket. Adding these two forces we get total chain tension to be ~5357N.

Velocity of small sprocket

Now we know that the chain has a tension of ~5357N. Well, this force is not acting on just one tooth and not all the engaged teeth distribute the force equally. In fact the force is the highest at the first eneged tooth on the 47T sprocket and then it decreases based on the back tension equation below:

Back tension equation

The back tension equation give you the force acting on each tooth under load:

Teeth under load

Now that we know our teeth under load and the force actin at each tooth on the 47T sprocket then we have our loading condition. To design the topology of the sprocket hub and the sprocket itself, we have to assume our worst possible case which is the lockup of the differential. This would mean the sprocket and hub would have to resist the loads applied by the motor which is the forces of those 22 teeth.

Doing some research I found that motorcycle sprockets and hubs generally have a factor of safety of 3 and above. This is another target of our for the topology design.

So where do we go from here?

There are a few things that must be considered when designing the hub and sprocket:

For the hub:

- Shear stress at every radius r on the hub

- Bearing yield at the location of the mounting bolts.

- Spline shear, and compression

For the sprocket:

- Bearing yield at the bolt locations

- Tooth shear stress

*Because of a time crunch and because we get out spline directly from Drexler which manufactures our sprocket we skipped the spline shear analysis.

HUB | Shear Stress

Ok well, we need to figure out where most of our stress is concentrated and design around this high stress area. If you take the sum of the forces on the first 11 teeth you find that this sum account for ~99% of the total force across all engaged teeth. So we can use this parameter and design a 11/47  or ~25% of the hub geometry since we know the force is localized mainly around these 11/47 teeth and then copy the geometry all around.

We have from last years geometry that the thickness of the hub must be 27mm to be able to perfectly fit the differential and that this thickness exists from 0mm<diameter<70mm. This means that we start topology optimization after that 70mm diameter or 35mm radius.

Step 1 is to find the moment acting on the hub about the axis. This can be found using Macting=Fr where F is the sum of the forces on those 11 teeth and r is the pitch circle of the sprocket. This will be our reference moment. We assume out spline is fixed and is unable to spin such that we can perform a static structural analysis.

Step 2. Is to derive an equation that will give us the force acting at any radius on the hub. We use the same equation to find F = Macting / r where r is the input radius from 35mm < r < Dmax/2

Step 3. We need to find the shear stress at any radius r. Shear stress is given by F/A where F in this case is the shear force. We found earlier that force is found by F = Macting / r . So what about area? Well we know that Area = Arc Length * thickness, let L = Arc Length and t = thickness. Arc length in this case is given by (11/47)*2*π*r.  inputting  F = Macting / r and A = (11/47)*2*π*r*t we get

τ = M / (11/47)*2*π*r2*t.

Step 4. Solve for thickness. Using our factor of safety requirement nd= Shear Strength /  τ ; we can derive an equation that can solve for our necessary thickness. Our necessary thickness. Let Shear strength = Sys . Using the maximum shear stress theory that says shear strength is half of tensile yield strength we find that solving for thickness give us the equation:

t = 6M / (11/47)*2*π*r2*Syt

Plugging in all our values (using a tensile strength value for 7075 T6 of 503MPa we get a minimum thickness of  ~6mm.

THIS THICNESS ASSUMES A SOLID HUB THROUGHOUT WITH NO CUTUOTS. WE NEED CUTOUTS TO BE ABLE TO IMPOVE THE HEAT DISSIPATION QUAITIES OF THE HUB, AND REDUCE THE AMOUNT OF PTETNIAL FLEXING GIVEN THE THIN CROSS SECTION.

To figure out the cutouts, we increase the minimum thickness value to 7.31mm (like last years which did not fail). So now we need to know how much material is needed at each radius. Going back to this equation: t = 6M / (11/47)*2*π*r2*Syt  We now have to solve for arc length. Because we are no longer assuming material about the whole arc length then our 11/47 value x becomes some arbitrary value we have to solve for and thickness becomes a known value. Solving for our value x we get x = 6M/2*π*r2*Syt*t. We know x can be interpreted as a fraction of 360 degrees, so we can say x = θ/360; solving for theta we get

θ = 360(6M/2*π*r2*Syt*t) .

We can know plot out the various thetas on excel at different radii to start drawing the topology for the hub.

Why were M10 bolts used? When we were doing the bearing yeidl calculation where stress = F / Dt where D is the diameter of the bore, we ran different iterations of the calculation testing different bolt sizes. The M10 bolts gave us an adequate factor of safety of 4.245. In these calculations we also identified that the local thickness had to be 9.23mm to get this factor of safety, this calculation was a lot of trial and error until we were able to fine tune a thickness and diameter that met the factor of safety requirement of 3 or higher.

FEA

The bolts themselves were placed under the same assumed conditions in an ASNYS structural analysis where the preload for the bolts was calculated (75% of bolt strength) and the spline was fixed and the teeth were subjected to the load given by the back tension equation. The results can be viewed below:

Current Status: I submitted a drawing of the 47T sprocket to our manufacturer and I am waiting on the response for the quote. The Sprocket hub will be machined inhouse using a 3 axis Haas CNC mill.